The enclosed programs lm4.exe and bc221.exe will generate either the LM or

BC-221 calibration books for those frequency meters lacking such a calibration

book. To generate the calibration book, first you must edit the proper data

file based on the unit you are calibrating. If you have a LM-1 thru 9, edit

the file "lmcalib.old", entering your data for the indicated frequencies.

If you have a LM-10 thru 22, edit the file "lmcalib.new". Finally for the

BC-221, edit the file "bc221.dat".  The data for your unit can be obtained

via several means. 1) Use a frequency counter to measure the units output

and obtain the corresponding dial settings.  2) Use a known LM or BC-221 to

obtain the dial setting for the desired frequencies.  3) Use a receiver with

crystal calibrator to beat the unknown unit frequency against a known

calibration point. Use a multiple of the units frequency

(i.e. 130 kc x 10 = 1300 kc) to beat with a known crystal marker

(i.e. 1300 kc).

After editing the data file with the correct information, rename the edited

"lmcalib.old" or "lmcalib.new" to "lmcalib.dat". Run the program lm4.exe or

bc221.exe to generate the calibration book.  The ".exe" program will generate

a new file called lmcalib.prn (for the LM frequency meter) or bc221cal.prn

(for the BC-221 frequency meter).  Copy the new ".prn" file to your printer

to obtain the the calibration book for your unit.

The enclosed ".exe" files should run on most IBM or compatable computers.

However, if the files will not run, you may have to recompile them for your

machine. The necessary Fortran files can be found in the subdirectory

"fortran".  Copy all the files to your own subdirectory. Run "setup.bat"

and follow the instructions, entering the necessary information relative

to your machine. The result will be a new ".lib" file tailored for

your machine.  The necessary fortran source files (lm4.for or

bc221.for) can be found in the appropiate subdirectory "lm4" or "bc221".

Copy the correct ".for" file into the same directory as the Fortran files.

Also copy the appropiate ".dat" file from above into the same directory.

To compile the new program, type "for1" at the prompt. You will then be asked

for the ".for" program name.  Enter "lm4" or "bc221", depending on your unit.

Hit "enter" (or "return") for the next three prompts. You should receive the

result "No errors" and the DOS prompt. Next enter "for2".  You should receive

a "No errors found" and the DOS prompt.  Enter "link".  You will then be asked

for the ".obj" file name, enter "lm4" or "bc221", depending on you program.

Hit "enter" (or "return") for the next three prompts.  At the DOS prompt,

enter "lm4" or "bc221" to run your new ".exe" file.  Assuming you entered

the required data correctly, you will get a new ".prn" file to send to your

printer to generate your new calibration book

The calibration book you generate will not be as accurate as an original

book.  However, the maximum error in the HF range should be less the 0.5 kc

at the ends of the range.  The maximim error in the LF range should be less

than 50 cycles at the ends of the range. This error is due to a small

nonlinearity in the capacitor manufacture.  Not bad for a 55 year old design

and 55 year old technology!

Enjoy.  73  Russ Schroeder  W2DYY  9 Feb 96